| الرقم | اسم المادة ومواصفاتها | الوحدة | الكمية المطلوبة | الماركة | المنشأ | مدة التسليم | الأسعار بالدينار الأردني |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | السعر الإفرادي |  | السعر الإجمالي |  |
|  |  |  |  |  |  |  | ف | د | ف | د |
| 1 | اسم المادة: Advanced Metallurgical Microscope with Polarizing Darkfield & Dual Lights الوصف: * An advanced metallurgical microscopy system with reflected illuminations and a USB digital camera with at least 5.0MP resolution. * Used for metallographic inspection of metals and alloys. * Magnification at least 1000x * Built-in bright/dark viewing field systems with conversion slide. * Must include a software for editing microscope images (grain analysis, phase detection, etc…) or recording video compatible with Microsoft windows. * Measuring length, angle, area, depth and etc. across microscope images. * Built-in reduction lens providing the same view on PC screen as that seen through eyepieces. * Easy USB PC connection. * 30o inclined 360o rotate-able compensation free trinocular head (or larger). | جهاز | 1 |  |  |  |  |  |  |  |
| 2 | اسم المادة: 12” Platens Grinder/ Polisher with Magnetic Plates & Complete Accessories الوصف: * Aluminum alloy master plate at least 10" dia lapping plate * At least 10" dia.× 0.8 mm thick permanent magnetic sheet with PSA backing (installed on Aluminum plate) * Two high torque motor with belt pulley with independent speed control from 0 - 600 RPM with digital display for each plate. * Manufacturer warranty | جهاز | 1 |  |  |  |  |  |  |  |
| 3 | اسم المادة: Precision Composites Curing Oven الوصف: * A fully featured industrial curing oven for convenient bench-top installation. * Chamber dimensions up to 1070×430×500 (W×D×H) mm * Maximum temperature in the 200 – 300 °C range * It is designed for the precision curing of prepreg moulds and components, casting resins, silicones and more. * The oven must include an intuitive touch screen controller with onboard data logging, additional auxiliary temperature sensor input and an automated vacuum pump control connection. * The oven must also include multiple shelf positions and two threaded vacuum port connections. * Manufacturer warranty | فرن | 1 |  |  |  |  |  |  |  |
| 4 | اسم المادة: Mobile Industrial Vacuum Pump الوصف: * An industrial mobile vacuum system designed for factory floor use. * Nominal Capacity 20-24 m3/h * Total Final Pressure 2 mbar * Total Final Pressure 99.98 % Abs. * Operating Temperature @ 20°C = 65-70°C * Warranty | مضخة | 1 |  |  |  |  |  |  |  |
| 5 | اسم المادة: Portable Vacuum Pump الوصف: * Compact, portable vacuum pump, engineered specifically for the rigours of daily use in a composite's environment. ideal for vacuum bagging, prepregs, resin infusion and vacuum degassing * Nominal Capacity (Metric): 4 m3/h * Total Final Pressure 99.98 % Abs. * Motor Power: 012 kW * Vacuum Port size: 1/4" (Fitted with an 8mm Hose Tail Barb) * Warranty | مضخة | 1 |  |  |  |  |  |  |  |
| 6 | اسم المادة: Vacuum Degassing System with full accessories الوصف: * Recommended Capacity 26 liters. * Total Final Pressure 99.95 % Abs. * Operating Temperature @ 20°C = 65-70°C * Warranty | جهاز | 1 |  |  |  |  |  |  |  |
| 7 | اسم المادة: Ultrasonic Processor الوصف: * Ultrasonic Processors to safely process a wide range of organic and inorganic materials * 750 Watt Ultrasonic Processor * Up to 1 litre capacity * Energy and temperature set-point control * Digital wattmeter * Up to 10 hour process timer * Variable power output control | جهاز | 1 |  |  |  |  |  |  |  |
| 8 | اسم المادة: General purpose data logger الوصف: * Contains 5 – 15 Channels * Expandable to 200 channels. * Voltage and Current input 4-20mA, Temperature, Humidity, Strain Gauges, Logic, Rotary Encoders & Serial Sensor. * Speed range from at least 100ms to Days. * PC Connection through Ethernet, USB & RS232. * Data transfer using USB memory device or direct to PC. * Internal battery Power of 2.5 hrs to 3 months or External. * Software included. * Manufacturer warranty | جهاز | 1 |  |  |  |  |  |  |  |